SUCTION HOSE

Task Force Tips Model # AM4FSX10 suction hose shall be provided. The suction hose shall have a clear PVC hose body to allow visibility of hose fill and remain flexible at low temperatures. The rigid black helix shall provide support and abrasion resistance. TFT's patent pending V-Seal, nylon binding strip and smooth contoured shanks shall be secured by dual stainless steel bands to prevent leakage and prevent hose damage under tight bending conditions. Minimum bending radius is 12-1/2". All aluminum parts are hardcoat anodized for maximum corrosion resistance. Hose is not intended for pressure service.

Suction hose shall be 10 feet in length, 6" ID and have 6" Storz swivel folding long handle couplings.