SUCTION HOSE

Task Force Tips Model # AM8FNX10 suction hose shall be provided. The suction hose shall have a clear PVC hose body to allow visibility of hose fill and remain flexible at low temperatures. The rigid black helix shall provide support and abrasion resistance. TFT's patent pending V-Seal, nylon binding strip and smooth contoured shanks shall be secured by dual stainless steel bands to prevent leakage and prevent hose damage under tight bending conditions. Minimum bending radius is 12-1/2". All aluminum parts are hardcoat anodized for maximum corrosion resistance. Female coupling shall have two folding handles that shall automatically fold back when not in use. Hose is not intended for pressure service.

Suction hose shall be 10 feet in length, 6" ID and have 6" female NH swivel folding long handle coupling and 6" male NH rigid rocker lug coupling.