
REMOTE MONITOR VALVE INTERFACE BOX
Task Force Tips model # YE-VK electric valve interface box for Tornado series remote control monitors shall be installed. The interface box is for connection of valve, joystick control, monitor and power. The interface box electrical components shall be compatible with 12 or 24 volt direct current.
The enclosure shall be weatherproof and utilize weatherproof components such as a silicone seal, hardware with o-rings and liquid tight electrical connections with strain relief fittings.
The enclosure shall include the following:
A preconnected 20 foot, four-conductor cable with waterproof, four-pin connection plug  for power and communications connection to the monitor;
A preconnected 20 foot, two conductor cable with waterproof, two-pin connection plug for connection to valve controller;
A preconnected 15 foot, two conductor cable with stripped ends for connection to vehicle power;
A four-pin receptacle with cap for connection to joystick control;
An unused strain relief fitting for an additional operator station cable.
The valve interface box shall be covered by a written five-year warranty.


